CS223 UART DESIGN PROJECT

Name: Barış Pamuksuz

ID: 22202238

Section: 1

TASK 1: DESIGN AND FUNCTIONALITY

Main UART (part 1) requires basically TX and RX buffers implementation.  To establish same bit sending-receiving, same period should be used. Period is determined by the baud rate, that I took 115200 in my code. Hence CLKS_PER_BIT is 100Mhz/115200 = 868, meaning that at the end of 868 cycle, new bit is sent or obtained. Both of buffers have 4 states. They are idle, start, get/send bit and stop states.  For TX buffer, when state is Idle, logic is 1. Whenever user press BTNC, TX goes to start state and sends 0 signal to RX buffer. Since RX buffer too was in idle state, incoming 0 signal initiates state cycling by moving start state to be able to receive a byte. RX and TX buffers move into get bit and send bit states respectively. After sending data, RX checks the parity bit generated by TX according to number of 1s. In my case, I used even parity. If income data holds even number of 1s and 0 as parity or odd number of 1s and 1 as parity, data receive is completed and transmitted to stop state that is in logic high signal. After a while, again, returned to idle state for both buffers.

In second part, FIFO registers are completed by  defining 32-bit memories for both buffers. For TX, when loadButton is pressed, new data is loaded to first 8 bits, that is TX[0], and TX[3] is discarded with the convention that rest of data are sided by one to the right. Discarded data is sent through state mechanism to RX. For RX, after get bit state, that is completing a 8 bit sequence successfully, obtained data is set to RX[0] and R[3] is discarded with the convention of moving income data by one to the right. Since button inputs might cause noise or unwanted input streams, I implemented button debounce to ensure pushing on button generates 1 signal not constantly.

In third part, to get hexadecimal values of data inside memories, I divided each byte into 2 nibbles. Each nibble corresponding to later displayed hexadecimal symbols. Other 2 digits are assigned to determine in which memory location 7-segment-display shows and the index of data inside those memories.

TASK 2: RTL SCHEMATICS

TX RTL

RX RTL

7SegmentDisplay RTL

TASK 3: STATE DIAGRAMS

RX

TX

TASK 4: BLOCK DIAGRAM FOR EACH MODULES

TASK 5: TESTBENCH RX – TX

TESTBENCH TX MODULE

`timescale 1ns / 1ps

module testbench;

reg [7:0] data_to_Transfer;

reg data_ready;

reg loadButton;

reg clock;

wire tx;

wire[7:0] mostRecent;  // Assuming this is also an output now

Uart_tx uart_tx_inst (

.data_to_Transfer(data_to_Transfer),

.data_ready(data_ready),

.loadButton(loadButton),

.clock(clock),

.tx(tx),

.mostRecent(mostRecent)          // Assuming these are correctly declared as outputs in the Uart_tx

);

// Clock generation

initial begin

clock = 0;

forever #5 clock = ~clock;

end

// Test sequence

initial begin

// Initialize inputs

data_to_Transfer = 0;

data_ready = 0;

loadButton = 0;

#100;

// Test 1: Load data into FIFO

@(posedge clock);

data_to_Transfer = 8'hAA; // Example data

loadButton = 1;

@(posedge clock);

loadButton = 0;

// Test 2: Load another data

@(posedge clock);

data_to_Transfer = 8'hBB; // Example data

loadButton = 1;

@(posedge clock);

loadButton = 0;

// Test 3: Load additional data while sending

repeat(10) @(posedge clock);

data_to_Transfer = 8'hDD;

loadButton = 1;

@(posedge clock);

loadButton = 0;

// Test 4

repeat(5) @(posedge clock);

data_to_Transfer = 8'hEE;

loadButton = 1;

@(posedge clock);

data_to_Transfer = 8'hFF;

@(posedge clock);

loadButton = 0;

repeat(10) @(posedge clock);

data_ready = 1;

@(posedge clock);

data_ready = 0;

#100

@(posedge clock);

loadButton = 1;

@(posedge clock);

loadButton = 0;

// Continue simulation to observe full behavior

#200;

// End simulation

$finish;

end

endmodule

TESTBENCH RX MODULE

`timescale 1ns / 1ps

module testbench2;

reg i_rx;

reg clock;

reg [7:0] memoryR;

Uart_rx uart_rx_inst (

.i_rx(i_rx),

.clock(clock),

.data_receivedOldest(memoryR)

);

// Clock generation

initial begin

clock = 0;

forever #5 clock = ~clock; // 100 MHz clock

end

// Simulate incoming data bits

initial begin

memoryR = 8'b0;

// Initialize

i_rx = 1;

#100;

// START_STATE bit

i_rx = 0;

#86.8;

// Data bits for 0x55 -> 01010101

i_rx = 1; #86.8;

i_rx = 0; #86.8;

i_rx = 1; #86.8;

i_rx = 0; #86.8;

i_rx = 1; #86.8;

i_rx = 0; #86.8;

i_rx = 1; #86.8;

i_rx = 0; #86.8;

i_rx = 1; #86.8;  // Parity (even)

i_rx = 1; #86.8;  // STOP_STATE bit

i_rx = 1; #173.6;

// Send 0xAA -> 10101010

i_rx = 0; #86.8;

i_rx = 0; #86.8;  // Bit 0

i_rx = 1; #86.8;  // Bit 1

i_rx = 0; #86.8;  // Bit 2

i_rx = 1; #86.8;  // Bit 3

i_rx = 0; #86.8;  // Bit 4

i_rx = 1; #86.8;  // Bit 5

i_rx = 0; #86.8;  // Bit 6

i_rx = 1; #86.8;  // Bit 7

i_rx = 0; #86.8;  // Parity bit (odd)

i_rx = 1; #86.8;  // STOP_STATE bit

i_rx = 1; #173.6; // IDLE_STATE state

// Send 0xFF -> 11111111 to test all high bits

i_rx = 0; #86.8;  // START_STATE bit

i_rx = 1; #86.8;  // Bit 0

i_rx = 1; #86.8;  // Bit 1

i_rx = 1; #86.8;  // Bit 2

i_rx = 1; #86.8;  // Bit 3

i_rx = 1; #86.8;  // Bit 4

i_rx = 1; #86.8;  // Bit 5

i_rx = 1; #86.8;  // Bit 6

i_rx = 1; #86.8;  // Bit 7

i_rx = 0; #86.8;  // Parity bit (even)

i_rx = 1; #86.8;  // STOP_STATE bit

i_rx = 1; #173.6; // IDLE_STATE state

i_rx = 0; #86.8;  // START_STATE bit

i_rx = 1; #86.8;  // Bit 0

i_rx = 0; #86.8;  // Bit 1

i_rx = 1; #86.8;  // Bit 2

i_rx = 0; #86.8;  // Bit 3

i_rx = 1; #86.8;  // Bit 4

i_rx = 0; #86.8;  // Bit 5

i_rx = 1; #86.8;  // Bit 6

i_rx = 0; #86.8;  // Bit 7

i_rx = 1; #86.8;  // Parity bit (even)

i_rx = 1; #86.8;  // STOP_STATE bit

i_rx = 1; #173.6; // IDLE_STATE state

i_rx = 0; #86.8;  // START_STATE bit

i_rx = 1; #86.8;  // Bit 0

i_rx = 0; #86.8;  // Bit 1

i_rx = 1; #86.8;  // Bit 2

i_rx = 0; #86.8;  // Bit 3

i_rx = 1; #86.8;  // Bit 4

i_rx = 0; #86.8;  // Bit 5

i_rx = 1; #86.8;  // Bit 6

i_rx = 0; #86.8;  // Bit 7

i_rx = 1; #86.8;  // Parity bit (even)

i_rx = 1; #86.8;  // STOP_STATE bit

i_rx = 1; #173.6; // IDLE_STATE state

$finish;

end

endmodule

APPENDIX

TX

module tx #(parameter CLKS_PER_BIT = 868)(

input logic clock,

input logic btn_load_out,

input logic transmit_button,

input logic [7:0] loaded_data,

output logic tx,

output logic [7:0] sent_data,

output logic [31:0] txMemory

);

logic parity_bit;

reg [15:0] counter = 0;

reg [8:0] sent2_data = 0;

localparam idle = 2'b00;

localparam start = 2'b01;

localparam send_bit = 2'b10;

localparam stop = 2'b11;

reg [1:0] state = 0;

reg [3:0] bit_index = 0;

always @(posedge clock) begin

case(state)

idle: begin

tx <=1;

counter <= 0;

bit_index <= 0;

if(btn_load_out)begin

txMemory[31:24] <= txMemory[23:16];

txMemory[23:16] <= txMemory[15:8];

txMemory[15:8] <= txMemory[7:0];

txMemory[7:0] <= loaded_data[7:0];

end

if(transmit_button) begin

sent2_data[7:0] <= txMemory[31:24];

sent_data[7:0] <= sent2_data[7:0];

parity_bit = sent2_data[0]^sent2_data[1]^sent2_data[2]^sent2_data[3]^

sent2_data[4]^sent2_data[5]^sent2_data[6]^sent2_data[7];

sent2_data[8] <= parity_bit;

txMemory[31:24] <= txMemory[23:16];

txMemory[23:16] <= txMemory[15:8];

txMemory[15:8] <= txMemory[7:0];

txMemory[7:0] <= 0;

state <= start;

end

else begin

state <= idle;

end

end

start: begin

tx <= 0;

if(counter < (CLKS_PER_BIT - 1)) begin

counter = counter + 1;

state <= start;

end

else begin

counter <= 0;

state <= send_bit;

end

end

send_bit: begin

tx <= sent2_data[bit_index];

//led2[7:0] <= sent2_data[7:0];

if(counter < (CLKS_PER_BIT - 1)) begin

counter <= counter + 1;

state <= send_bit;

end

else begin

counter <= 0;

if(bit_index < 8) begin

bit_index <= bit_index + 3'b1;

state <= send_bit;

end

else begin

bit_index <= 0;

state <= stop;

end

end

end

stop: begin

tx <= 1;

if(counter < (CLKS_PER_BIT - 1)) begin

counter= counter + 1;

state <= stop;

end

else begin

state <= idle;

end

end

default:

state <= idle;

endcase

end

endmodule

RX

module rx#(parameter CLKS_PER_BIT = 868)(

input logic clock,

input logic input_rx,

output logic [7:0] rx_out,

output logic [31:0] memory_rx

);

logic parityBit;

localparam IDLE = 2'b00;

localparam START = 2'b01;

localparam GET_BIT_STATE = 2'b10;

localparam STOP_STATE = 2'b11;

reg rxBuff = 1'b1;

reg rx =  1'b1;

reg [1:0] state = 0;

reg [15:0] counter = 0;

reg [8:0] data_received = 0;

reg [3:0] bit_index = 0;

always @(posedge clock) begin

rxBuff <= input_rx;

rx <= rxBuff;

end

always @(posedge clock) begin

case(state)

IDLE: begin

counter <= 0;

bit_index <= 0;

if(rx == 0)

state <= START;

else

state <= IDLE;

end

START: begin

if(counter == (CLKS_PER_BIT-1)/2) begin

if(rx == 0) begin

counter <= 0;

state <=GET_BIT_STATE;

end

else begin

state <= IDLE;

end

end

else begin

counter <= counter + 1;

state <= START;

end

end

GET_BIT_STATE: begin

data_received[bit_index] <= input_rx;

if(counter < (CLKS_PER_BIT-1)) begin

counter <= counter + 1;

state <= GET_BIT_STATE;

end

else begin

counter <= 0;

if(bit_index < 8) begin

bit_index <= bit_index +3'b1;

state <= GET_BIT_STATE;

end

else begin

rx_out[7:0] <= data_received[7:0];

parityBit = data_received[0]^data_received[1]^data_received[2]^data_received[3]^

data_received[4]^data_received[5]^data_received[6]^data_received[7];

if(parityBit != data_received[8])begin

data_received <= 0;

end

bit_index <= 0;

state <= STOP_STATE;

end

end

end

STOP_STATE: begin

if(counter < CLKS_PER_BIT-1) begin

counter <= counter + 16'b1;

state <= STOP_STATE;

end

else begin

memory_rx[31:24] <= memory_rx[23:16];

memory_rx[23:16] <= memory_rx[15:8];

memory_rx[15:8] <= memory_rx[7:0];

memory_rx[7:0] <= data_received[7:0];

counter <= 0;

state <= IDLE;

end

end

default: state <= IDLE;

endcase

end

endmodule

Debounce

module debounce(

input logic clock,

input logic ButtonIn,

output logic Button_out

);

logic prevButtonIn = 0;

always_ff @(posedge clock)begin

prevButtonIn <= ButtonIn;

if(ButtonIn && !prevButtonIn)begin

Button_out <= 1;

end

else begin

Button_out <= 0;

end

end

endmodule

ButtonupLogic

module ButtonUpLogic(

input logic clock,

input logic btn_out,

output logic InTMemory

);

always_ff @(posedge clock)begin

if(btn_out)begin

InTMemory <= !InTMemory;

end

end

endmodule

DisplayMemory

module displayMemory(

input logic clock,

input logic rightButtonOut,

input logic leftButtonOut,

input logic [31:0] txMemory,

input logic [31:0] rxMemory,

output reg [3:0] anodes,

output logic [6:0] segments,

input logic r_is_true

);

logic [1:0] case_index = 0;

logic [1:0] counter;

logic [19:0] refresh = 20'b0;

logic [1:0] bit_index_tx;

logic [1:0] bit_index_rx;

logic [7:0] regDisplay;

reg [3:0] digit0;

reg [3:0] digit1;

reg [3:0] digit2;

reg [4:0] digit3;

reg [4:0] led = 0;

always @(posedge clock)

begin

digit0 <= regDisplay[3:0];

digit1 <= regDisplay[7:4];

if(r_is_true)begin

digit2 <= bit_index_rx;

digit3 <= 16;

end

else begin

digit2 <= bit_index_tx;

digit3 <= 17;

end

if(r_is_true && (bit_index_rx == 0))begin

regDisplay[7:0] <= rxMemory[7:0];

end

if(r_is_true && (bit_index_rx == 1))begin

regDisplay[7:0] <= rxMemory[15:8];

end

if(r_is_true && (bit_index_rx == 2))begin

regDisplay[7:0] <= rxMemory[23:16];

end

if(r_is_true && (bit_index_rx == 3))begin

regDisplay[7:0] <= rxMemory[31:24];

end

if(!r_is_true && (bit_index_tx == 0))begin

regDisplay[7:0] <= txMemory[7:0];

end

if(!r_is_true && (bit_index_tx == 1))begin

regDisplay[7:0] <= txMemory[15:8];

end

if(!r_is_true && (bit_index_tx == 2))begin

regDisplay[7:0] <= txMemory[23:16];

end

if(!r_is_true && (bit_index_tx == 3))begin

regDisplay[7:0] <= txMemory[31:24];

end

if(r_is_true && rightButtonOut)begin

bit_index_rx <= bit_index_rx+1;

end

if(!r_is_true && rightButtonOut)begin

bit_index_tx <= bit_index_tx+1;

end

if(r_is_true && leftButtonOut)begin

bit_index_rx <= bit_index_rx-1;

end

if(!r_is_true && leftButtonOut)begin

bit_index_tx <= bit_index_tx-1;

end

end

always @(posedge clock)

begin

if (counter > 1)

counter <= 2'b00;

else

counter <= counter + 1;

refresh <= refresh + 1;

case_index <= refresh[19:18];

end

always_ff @(posedge clock)

begin

case (case_index)

0: begin anodes = 4'b1110; led = digit0; end

1: begin anodes = 4'b1101; led = digit1; end

2: begin anodes = 4'b1011; led = digit2; end

3: begin anodes = 4'b0111; led = digit3; end

endcase

end

always_ff @(posedge clock)

begin

case (led)

0 : segments = 7'b0000001;

1 : segments = 7'b1001111;

2 : segments = 7'b0010010;

3 : segments = 7'b0000110;

4 : segments = 7'b1001100;

5 : segments = 7'b0100100;

6 : segments = 7'b0100000;

7 : segments = 7'b0001111;

8 : segments = 7'b0000000;

9 : segments = 7'b0000100;

10 : segments = 7'b0001000; // A

11 : segments = 7'b1100000; // B

12 : segments = 7'b1110010; // C

13 : segments = 7'b1000010; // D

14 : segments = 7'b0110000; // E

15 : segments = 7'b0111000; // F

16 : segments = 7'b1111010; // r

17 : segments = 7'b1110000; // t

default : segments = 7'b1111111;

endcase

end

endmodule

TopModule

module top_module(

input logic clock,

input logic loadButton,

input logic transmitButton,

input logic rightButton,

input logic leftButton,

input logic inRDisplay,

input logic [7:0] loaded_data,

output logic [7:0] received_first,

output logic [7:0] discarded_sent,

output logic [3:0] anodes,

output logic [6:0] seg

);

wire connection;

logic [31:0] rxMemory;

logic [31:0] txMemory;

logic LoadButtonOut;

logic TransmitButtonOut;

logic RightButtonOut;

logic LeftButtonOut;

debounce d1(clock, loadButton, LoadButtonOut);

debounce d2(clock,transmitButton,TransmitButtonOut);

debounce d3(clock, leftButton, LeftButtonOut);

debounce d4(clock,rightButton,RightButtonOut);

debounce d5(clock,inRDisplay,inRDisplay_out);

ButtonUpLogic buttonUpLog(clock,inRDisplay_out,ButtonUpLogic);

displayMemory s1(clock, RightButtonOut , LeftButtonOut , txMemory, rxMemory, anodes, seg, ButtonUpLogic);

tx sender(clock, LoadButtonOut, TransmitButtonOut, loaded_data, connection, discarded_sent, txMemory);

rx receiver(clock, connection, received_first, rxMemory);

endmodule